# Design of an 8-Bit Accumulator-Based Microprocessor

Submitted By:

صموئيل مرزوق ناجح مرزوق

عمرو سعد عبد الجابر محمد

محمد طه عبد العزيز توني

هيا محمود محمد محمد

خالد حمدي الداودي

Under Supervision of: Eng. Mohamed Salah Abo Shosha

Date: September 2026

## 1. Introduction

This report details the design, implementation, and verification of an advanced 8-bit Accumulator-based Central Processing Unit (CPU). The processor is designed using Verilog HDL at the Register-Transfer Level (RTL). The architecture features a Harvard memory model, separating instruction and data memory to enhance structural clarity and datapath efficiency. Furthermore, the CPU integrates a dedicated Status Register to handle arithmetic flags (Carry, Zero, Negative), enabling complex conditional branching operations.

## 2. System Architecture

The microprocessor is composed of several critical hardware modules integrated within a top-level structural wrapper (cpu_top.v):

- ALU (Arithmetic Logic Unit): Performs 8 distinct operations including arithmetic, bitwise logic, and shift operations.

- Status Register: Captures ALU state flags to inform the Control Unit for branching.

- Control Unit: A combinational decoder responsible for instruction decoding and orchestrating internal control signals.

- Harvard Memory Subsystem: Independent read-only memory (ROM) for instructions and read/write memory (RAM) for data.

- Sequential Elements: An 8-bit Accumulator for primary data manipulation and a 4-bit Program Counter (PC) for instruction tracking.

## 3. RTL Module Implementations

### 3.1. Arithmetic Logic Unit (ALU.v)

The ALU is the computational core, enhanced to output state flags for advanced decision-making.

module alu (

input wire [7:0] acc_in,

input wire [7:0] data_in,

input wire [2:0] alu_op,

output reg [7:0] alu_out,

output reg C,

output reg Z,

output reg N

);

reg [8:0] temp_add;

always @(*) begin

C = 1'b0;

case (alu_op)

3'b000: begin

temp_add = acc_in + data_in;

alu_out = temp_add[7:0];

C = temp_add[8];

end

3'b001: alu_out = acc_in - data_in;

3'b010: alu_out = acc_in * data_in;

3'b011: alu_out = acc_in & data_in;

3'b100: alu_out = acc_in | data_in;

3'b101: alu_out = acc_in ^ data_in;

3'b110: alu_out = acc_in << 1;

3'b111: alu_out = data_in;

default: alu_out = 8'h00;

endcase

Z = (alu_out == 8'h00) ? 1'b1 : 1'b0;

N = alu_out[7];

end

endmodule

### 3.2. Status Register (Status_Register.v)

module status_register (

input wire clk,

input wire reset,

input wire update_flags,

input wire alu_C,

input wire alu_Z,

input wire alu_N,

output reg C,

output reg Z,

output reg N

);

always @(posedge clk or posedge reset) begin

if (reset) begin

C <= 1'b0; Z <= 1'b0; N <= 1'b0;

end else if (update_flags) begin

C <= alu_C; Z <= alu_Z; N <= alu_N;

end

end

endmodule

### 3.3. Control Unit (Control_Unit.v)

The Control Unit is designed as a fully combinational block, implementing conditional branching capabilities based on state flags.

module control_unit (

input wire [3:0] opcode,

input wire flag_Z,

input wire flag_C,

output reg pc_inc,

output reg pc_load,

output reg acc_load,

output reg mem_write,

output reg [2:0] alu_op

);

localparam LOAD = 4'b0000;

localparam ADD  = 4'b0001;

localparam SUB  = 4'b0010;

localparam SAVE = 4'b0011;

localparam JUMP = 4'b0100;

localparam JZ   = 4'b0101;

localparam JC   = 4'b0110;

localparam HALT = 4'b1111;

always @(*) begin

pc_inc = 1'b0; pc_load = 1'b0; acc_load = 1'b0;

mem_write = 1'b0; alu_op = 3'b000;

case (opcode)

LOAD: begin acc_load = 1'b1; pc_inc = 1'b1; alu_op = 3'b111; end

ADD:  begin acc_load = 1'b1; pc_inc = 1'b1; alu_op = 3'b000; end

SUB:  begin acc_load = 1'b1; pc_inc = 1'b1; alu_op = 3'b001; end

SAVE: begin mem_write = 1'b1; pc_inc = 1'b1; end

JUMP: begin pc_load = 1'b1; end

JZ:   begin

if (flag_Z) pc_load = 1'b1;

else pc_inc = 1'b1;

end

JC:   begin

if (flag_C) pc_load = 1'b1;

else pc_inc = 1'b1;

end

HALT: begin

end

default: pc_inc = 1'b1;

endcase

end

endmodule

### 3.4. Memory Subsystem (Harvard Architecture)

Instruction Memory (Instruction_Memory.v)

module instruction_memory (

input wire [3:0] address,

output wire [7:0] instruction

);

reg [7:0] rom [0:15];

initial begin

rom[0] = 8'b0000_1010;

rom[1] = 8'b0010_1011;

rom[2] = 8'b0011_1010;

rom[3] = 8'b0101_0101;

rom[4] = 8'b0100_0000;

rom[5] = 8'b1111_0000;

end

assign instruction = rom[address];

endmodule

Data Memory (Data_Memory.v)

module data_memory (

input wire clk,

input wire write_enable,

input wire [3:0] address,

input wire [7:0] data_in,

output wire [7:0] data_out

);

reg [7:0] ram [0:15];

initial begin

ram[10] = 8'd3;

ram[11] = 8'd1;

end

always @(posedge clk) begin

if (write_enable) ram[address] <= data_in;

end

assign data_out = ram[address];

endmodule

### 3.5. System Core Registers

Accumulator (Accumulator.v)

module accumulator (

input wire clk,

input wire reset,

input wire acc_load,

input wire [7:0] data_in,

output reg [7:0] acc_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

acc_out <= 8'h00;

end else if (acc_load) begin

acc_out <= data_in;

end

end

endmodule

Program Counter (Program_Counter.v)

module program_counter (

input wire clk,

input wire reset,

input wire pc_inc,

input wire pc_load,

input wire [3:0] load_addr,

output reg [3:0] pc_out

);

always @(posedge clk or posedge reset) begin

if (reset) begin

pc_out <= 4'h0;

end else if (pc_load) begin

pc_out <= load_addr;

end else if (pc_inc)begin

pc_out <= pc_out + 1'b1;

end

end

endmodule

### 3.6. Top Module (CPU_Top_Module.v)

module cpu_top (

input wire clk,

input wire reset

);

wire [3:0] pc_out;

wire [7:0] instruction, memory_data_out, alu_result, acc_out;

wire pc_inc, pc_load, acc_load, mem_write;

wire [2:0] alu_op;

wire alu_C, alu_Z, alu_N, flag_C, flag_Z, flag_N;

wire [3:0] opcode = instruction[7:4];

wire [3:0] address = instruction[3:0];

program_counter PC (.clk(clk), .reset(reset), .pc_inc(pc_inc), .pc_load(pc_load), .load_addr(address), .pc_out(pc_out));

instruction_memory ROM (.address(pc_out), .instruction(instruction));

data_memory RAM (.clk(clk), .write_enable(mem_write), .address(address), .data_in(acc_out), .data_out(memory_data_out));

control_unit CU (.opcode(opcode), .flag_Z(flag_Z), .flag_C(flag_C), .pc_inc(pc_inc), .pc_load(pc_load), .acc_load(acc_load), .mem_write(mem_write), .alu_op(alu_op));

accumulator ACC (.clk(clk), .reset(reset), .acc_load(acc_load), .data_in(alu_result), .acc_out(acc_out));

status_register SR (.clk(clk), .reset(reset), .update_flags(acc_load), .alu_C(alu_C), .alu_Z(alu_Z), .alu_N(alu_N), .C(flag_C), .Z(flag_Z), .N(flag_N));

alu ALU_UNIT (.acc_in(acc_out), .data_in(memory_data_out), .alu_op(alu_op), .alu_out(alu_result), .C(alu_C), .Z(alu_Z), .N(alu_N));

endmodule

## 4. Verification & Testbench Environment

A self-checking verification methodology was utilized to validate the CPU logic autonomously without relying strictly on manual waveform inspection.

Testbench (tb_cpu.v)

`timescale 1ns/1ps

module tb_cpu;

reg clk;

reg reset;

cpu_top UUT (.clk(clk), .reset(reset));

initial begin

clk = 0; forever #5 clk = ~clk;

end

initial begin

$display("========================================");

$display("Starting Advanced Verification (Loop Test)");

$display("========================================");

reset = 1; #15; reset = 0;

#250;

if (UUT.RAM.ram[10] == 8'd0) begin

$display(">> [PASS] Loop executed successfully. Counter reached 0.");

end else begin

$display(">> [FAIL] Loop failed. Final Counter = %d", UUT.RAM.ram[10]);

end

$display("Simulation Complete.");

$stop;

end

endmodule

## 5. Simulation Waveforms

### 5.1. Initialization and Datapath Activity

### 5.2. Conditional Branching Execution

## 6. Conclusion

The implemented 8-bit architecture successfully demonstrates the core principles of digital IC design. By migrating to a Harvard memory structure and implementing a condition-aware data path, the processor can handle loops and branching logic efficiently. The logic synthesis simulation confirmed that all timing constraints and instruction executions operate reliably on clock edges.